I think you've just identified something more important than carry-forward.

The real issue is not:

"Should FG shortfall be added to next RS?"

The real issue is:

"What happened to the RM that was already issued?"

Example

RS:

FG Required = 5000 Caps
BOM = 4 Kg per 1000 Caps

RM Issued = 20 Kg

Production result:

FG Produced = 4000 Caps

Expected RM consumption:

16 Kg

Balance:

20 - 16 = 4 Kg

Now Production must do one of two things.

Case 1 — RM Returned

RM Issued = 20 Kg
RM Consumed = 16 Kg
RM Returned = 4 Kg

Store receives 4 Kg back.

No problem.

Next RS can issue fresh RM again.

Case 2 — RM Not Returned

RM Issued = 20 Kg
RM Consumed = 16 Kg
RM Returned = 0 Kg

Then:

Unexplained RM = 4 Kg

Production cannot simply close WO.

They must explain:

Machine setting loss
Startup loss
Material purging
QC rejection
Tool setting
Operator error
Material damaged
Other

This becomes:

RM Wastage = 4 Kg

and Store accepts it.

What This Means

FG carry-forward and RM accountability are two separate processes.

Process A — Production Planning

RS = 5000
Produced = 4000

FG Shortfall = 1000

This 1000 may go to next cycle.

Process B — RM Reconciliation

RM Issued = 20 Kg
RM Consumed = 16 Kg

Balance = 4 Kg

Production must:

Return RM
OR
Declare Wastage

before WO closure.

My Recommendation

Later in ERP we should introduce an RM Reconciliation Sheet at WO completion.

Something like:

| Item | Qty |
| --- | --- |
| RM Issued | 20 Kg |
| Expected Consumption | 16 Kg |
| Actual Consumption | 16 Kg |
| RM Returned | 3 Kg |
| RM Wastage | 1 Kg |
| Reason | Purging during color change |

Validation:

RM Issued
=
Actual Consumption
+
Returned
+
Wastage

otherwise WO cannot close.

This is actually closer to how plastic moulding factories work.

Store's responsibility ends at:

Material Issue

Production's responsibility begins at:

Material Accountability

and every issued kilogram must eventually become:

FG
OR Returned RM
OR Approved Wastage

That gives you a much stronger inventory control system than relying on FG carry-forward alone.